USAGE AND MAINTENANCE MANUAL

ELE

ELECTRIC CONVECTION OVEN

PRIME061EM                                       PRIME101EM

PRIME102EM                                       PRIME202EM

# PRESENTATION

DEAR CUSTOMERS

- To get the best yield of our products that are in compliance with the International Standards, we highly recommend you to read the user’s manual and have it read to your users for long and continuous usages.

- This manual contains important informations about the secure installation, usage and maintenance of the product and necessary remarks to make the most effidicent usage of your device.

- Keep this manual in an easily accessible and safe place to use when necessary.

- Our product is designed for heating and cooking food commodities. Any chemical or liquid substances or similar that can be in contact with the foodstuff, cannot be used for any heat treating that can cause damages for the health.

- The manufacturer cannot be held responsible for any damages from incorrect usage of the device, to people, the environment, or other materials resulting from translation or printing of this booklet.

# INDEX

PRESENTATION	2

INDEX	3

SAFETY DETAILS	4

TECHNICAL SPECIFICATIONS	5

TRANSPORTATION, PLACEMENT AND READY TO INSTALL	7

ELECTRICAL CONNECTION	8

CONTROL PANEL	9

1-) Switching On	10

2-) Switching Off	10

CLEANING	11

MAINTENANCE AND MALFUNCTION	11

Problems and Solutions	11

WARRANTY CONDITIONS	12

Out of Warranty Cases	13

AUTHORIZED SERVICE START-UP PAGE	15

Start Up Page	15

# SAFETY DETAILS

- Do not expose the device to direct sunlight.

- Any kind of flammable or ignitiable solid or liquid such as alchocol and derivatives, petro-chemical products, wood-plastic materials, curtains, clothes, etc. should not be kept in the area of the device.

- This device must be installed in accordance with the current regulations and should only be used in a well-ventilated area.

- The instructions should be analysed before the installation and the usage of the device.

- The device is designed for professional usage and should only be used by trained personnel.

- It should not be interfered without the presence of the manufacturer or the authorized service.

- The device or its components cannot be used for any other purpose.

- Apart from the heating and the cooking of the food, it cannot be used as a heater for any purpose.

- The device cannot be left uncontrolled as long as it is on.

- Food compatible gloves must be used to protect hands from the overheating.

- In case of fire or flame in the area where the device is, switch off the safety net which is located in the main circuit connection and use the fire extinguisher. Water should absolutely not be used, otherwise the flame will sprawl faster.

- The installation and the maintenance should be carried out by the qualified staff autorized by the manufacturer. Serious danger for the user can be arise due to incorrect installations, maintenances or repairs, and the manufacturer cannot be held responsible.

# TECHNICAL SPECITICATIONS

| PRODUCT CODE |  |  | PRIME061EM | PRIME101EM | PRIME102EM | PRIME202EM |
| --- | --- | --- | --- | --- | --- | --- |
| DIMENSIONS (mm) | A | WIDTH | 970 | 970 | 1200 | 1200 |
|  | B | DEPTH | 910 | 910 | 1100 | 1100 |
|  | C | HEIGTH | 865 | 1155 | 1155 | 1845 |
| ELECTRIC POWER |  |  | 12,65 kW | 18,65 kW | 27,65 kW | 55,3 kW |
| BLOWER POWER |  |  | 650 W | 650 W | 650 W | 650X2=1300 W |
| ELECTRIC INLET |  |  | 380-400 VAC 3N PE 50 HZ |  |  |  |
| CABLE CROSS SECTION |  |  | 5*4 mm2 | 5*4 mm2 | 5*6 mm2 | 5*16mm2 |
| WEIGHT |  |  | 115 KG | 130KG | 155 KG | 250 KG |

B

A

910

865

C

PRİME061EM

910

B

A

1155

C

PRİME101EM

B

1100

A

C

1155

PRİME102EM

B

1100

A

1845

C

PRİME202EM

# TRANSPORTATION, PLACEMENT AND READY TO INSTALL

- Do not tilt the device during the transportation, carry it upright.

- Use a suitable vehicle to avoid any kind of damages.

- Do not push or drag the device to bear.

- When transporting by pallet jack or forklift, make sure to not damage the device.

- Carry the device with its packaging until the installation area.

- It should be placed under a ventilation system to avoid any kind of bad smell that may occur during cooking process.

- It is recommended to place a suitable sized hood for the ventilation over the device without blocking the outlet of the chimney.

- In case the device is installed next to a wall, a partition, a kitchen furniture etc., make sure they are made by non flammable material or with a suitable thermal insulation coating with a margin of 10 cm, otherwise with a margin of 30 cm.

- Never place a combustible material on top of the device.

- Make a general cleaning before the first usage. You may use soapy water.

- The operating environment and the ventilation system of the device must be between 10°C to 40°C and have a maximum humidity level of %80.

- The device must be installed in accordance within the instructions.

- The protective film on the surface of the device must be stripped off carefully.

- Convenient electrical connection with the required technical specifications must be installed in the area of the device.

# ELECTRICAL CONNECTION

- Technical specification section should be analysed for the appropriate cable to be used for the electrical connection of the device.

- The electrical supply circuit must be done by the authorized staff. Before installation, make sure that your fitment meets the requirements. The device should absolutely not be used without grounding.

- The electrical voltage must be stable for the proper fonction of the device. If the malfunction is caused by the voltage, the replacement of these parts are considered “out of warranty”. Fault detection is based on the service declaration.

To prevent the device from any short circuit and overcurrent, the device must be supplied with the appropriate fuse control

PRİME101EM

PRİME061EM

PRİME102EM

PRİME202EM

EI= ELECTRIC INLET

W= WATER INLET

# CONTROL PANEL

PRIME061EM                       PRIME101EM                 PRIME102EM-PRIME202EM

# USAGE

- The device is designed for professional usage and should only be used by trained personnel.

MODE SELECTION BUTTON

THERMOSTAT BUTTON

THE

HUMIDITY BUTTON

TIMER

## 1-) Switching On

Convection Mode

- Turn on the fuse to which the device is connected.

- Turn the mode selector switch located on the control panel to clockwise or two stages to counterclockwise to the convection mode. There will be an aural warning. This warning will stop when the required time is adjusted.

- After setting the mode, adjust the time with the timer knob.

- The cooking process will start after closing the door.

- At the end of the cooking period, the oven will give an aural warning.

Half Humidity Mode

- Turn on the fuse to which the device is connected.

- Open the water valve to which the device is connected.

- Turn the mode selector switch located on the control panel 3 stages to clockwise or one stage to counterclockwise to the half humidity mode. There will be an aural warning. This warning will stop when the required time is adjusted.

- After setting the mode, adjust the time with the timer knob.

- Turn the steam supply knob to clockwise to give the required amount of steam. To reach that, the cabinet of the oven must be between 85°C – 130°C.

- The cooking process will start after closing the door.

- At the end of the cooking period, the oven will give an aural warning.

Cooling Mode

- Turn the mode selector switch to clockwise for 1 stage. When the required time is set with the time adjusting knob, the aural warning will stop.

- The cooking process will begin after adjusting the required time. At the end of this process, the oven will give an aural warning.

## 2-) Switching Off

- To turn off the device, set the mod selector button to the “0” position.

- Cihaz ile işiniz bittiğinde enerji beslemesinin yapıldığı sigortayı kapatarak emniyetinizi alınız.

# CLEANING

- Disconnect the wiring of the device before starting the cleaning process.

- When cleaning the device, make sure to prevent water from entering the electrical parts.

- Do not wash the device with high pressured water.

- After each operation, clean the device with a cloth dipped in warm soapy water.

- Do not use abrasive detergent or wire brushes when cleaning the device’s surface.

- Clean completely the device on specific periods with convenient cleaning materials.

# MAINTENANCE AND MALFUNCTION

- All connections and repairs must be carried out by the authorized service centers, otherwise, your device will be out of warranty.

## Problems and Solutions

| PROBLEM | SOLUTION |
| --- | --- |
| There is an aural warning and “Err” sign on the display panel. | The thermocouple that mesures the temperature of the cabinet is broken or there is a disconnection. Please contact the AUTHORIZED SERVICE. |
| The oven does not reach the required temperature. | There is a problem with the oven resistances or with the temperature control system. Please contact the AUTHORIZED SERVICE. |
| The oven is operating even though the door is open. | Check that the oven is not on cooling mode. If the oven is not on cooling mode, there is a problem with the door’s magnetic switch. Please contact the AUTHORIZED SERVICE. |
| The oven does not give an aural warning at the end of the adjusted time period. | There may be a problem with the buzzer or with its connection. Please contact the AUTHORIZED SERVICE. |
| In the half steam mode,even though the steam button is on, no steam is generated inside the oven. | Check the water connection valve. If the valve is open and no steam is generated, please contact the AUTHORIZED SERVICE. |

# WARRANTY CONDITIONS

- According to the consumer rights law and commercial law of the Ministry of Industry, except in a special case of agreement, sold industrial products by our company has one (1) year of warranty and that period starts from the delivery date of the product.

- The warranty includes all the components of the device against fabrication or faulty workmanship, eliminations of the problems or malfunctions of the device caused by the workmanship and installation errors, replacement of non-repairable parts or the mending of repaired parts that can be reused. It also includes repairings or changes of the components by the authorized personnel, or the labor fee due to the problem under the warranty.

- In order for the warranty conditions to be valid, the maintenance of the components fixed to the device, the maintenance of the product, as indicated in the user’s manual, should be done yearly and montly and the information form must be held after the maintenance to be presented when required.

- The document of the replacement of the component has to be presented for parts warranty claims.

- In order to continue the warranty period, the paid periodic maintenance has to be done by the authorized service and the information form must be held to be presented when required.

- In case of any malfunctions under the warranty period, the time spent in the repairment of the device is added to the warranty period. The repair can be maximum 30 (thirty) days. This period starts when the device is on the service station. In case of the absence of this station, the period will start when the manufacturer or the importer is notified by the product seller.

- In accordance with the regulations of the warranty document implementation fundamentals published by the Ministry of Industry, the warranty deed of the industrial products are the purchase invoices.

- If there is any kind of malfunctions due to both the materail and the workmanship, or assembly faults under the warranty period, the necessary replacement and reparing will be done by our authorized personnel without any charge.

## Out of Warranty Cases

- If the product is used above its intended purpose, standards or capacity specified by the manufacturer,

- In case of the bad locationing of the product, continuous usage and unhealthy environment (ex: required ventilation), not suitable floor or flat platform, lack of convenient gas pressure,

- If the product hasn’t been used as explained in the instruction manual,

- If the fault is caused by inconvenient, worng or careless usage,

- If the malfunction is caused because of fluctuations in mains electricity, lightning, fire, flood, earthquake and similar natural disasters,

- If the special tools and equipments installed or the maintenance and repair are made by any other person than the authorized person,

- When requesting a service, if the installation-start documents or purchase invoices of the product are not presented,

- If the informations on the documents of the installation, start or maintenance has been destroyed or changed,

- If the paid annual or the contractual periodic maintenance of the product has not been done on time by the authorized personnel,

- If unoriginal spare parts have been used,

- Notifications to our authorized services under the warranty period about the malfunctions that could not have been repaired, will make the time-out of the warranty stop only for that specific malfunction. In this case, the authorized service will inform about the compensation or the non-troubleshooting and the expanded warranty period will end in 3 months in any cases,

- Parts that may have possible abrasion in time such as mouldings, gas burners, lids, adjustmen knobs, flame holder thermocouples etc. will be repaired or changed with a fee.

- Damages caused by the unnecessary loss of time, the maleficience, the revenue loss, or the conduction of the product from the defective location to another,

- Malfunctions of the product due to squalidity are not covered by the warranty,

- The methods to be followed about the fault detection and elimination are determied completely by MUTAŞ’s technical service management and the service matter is taken as basis for the determination of the malfunction.

- In case of detection of the devices with any kind of fluid, warranty conditions will not be valid.

# AUTHORIZED SERVICE START-UP PAGE

## Start Up Page

- The model and serial number of your device are shown on the “Technical Data” label affixed to the device.

- Select the technical data according to the device model on the nameplate.

MUST BE FILLED BY AUTHORIZED SERVICE!

| COMPANY INFORMATIONS | : |
| --- | --- |
| PRODUCT | : |
| MODEL | : |
| SERIAL NUMBER | : |
| PRODUCTION DATE | : |
| SERVICE STAMP AND SIGNATURE | : |
| PERSONNEL OF THE SERVICE | : |
| CONTACT |  |
| TEL | : 0312 837 76 72 |
| FAX | : 0312 837 73 38 |
| E-MAIL | : teknikservis@vitalmutfak.com |

CONTACT

MUTAŞ MUTFAK ISITMA SİSTEMLERİ SAN. PAZ. İNŞ. TUR. İÇ VE DIŞ TİC.LTD.ŞTİ HEADQUARTER ADRES: Plevne Cad. No:40 Gülveren / ANKARA TEL: (0312) 363 99 33 FAX: (0312) 363 94 92 Web: www.vitalmutfak.com E-Mail:info@vitalmutfak.com

MUTAŞ MUTFAK ISITMA SİSTEMLERİ SAN. PAZ. İNŞ. TUR. İÇ VE DIŞ TİC.LTD.ŞTİ FACTORY ADRES: Taşpınar Mahallesi Küme Evleri No:154/A Çubuk / ANKARA TEL: (0312) 837 76 72 FAX: (0312) 837 73 38 Web: www.vitalmutfak.com E-Mail:teknikservis@vitalmutfak.com

AFTER SALES SERVICE

For the nearest MUTAŞ/VİTAL Authorized Service, please contact the numbers above.

teknikservis@vitalmutfak.com